# Circuit Pin Table

File: hardware/pin_table.md

Use this as a documentation draft. Verify every pin against the final ESP32 sketch and the physical wiring before connecting power.

| ESP32 pin / rail | Connects to | Notes |
| --- | --- | --- |
| GPIO 26 | L298N IN1 | Motor direction input; verify against firmware. |
| GPIO 27 | L298N IN2 | Motor direction input; verify against firmware. |
| GPIO 25 | L298N IN3 | Motor direction input; verify against firmware. |
| GPIO 33 | L298N IN4 | Motor direction input; verify against firmware. |
| GPIO 5 | HC-SR04 TRIG | Trigger output. |
| GPIO 18 | HC-SR04 ECHO through level divider | Never connect a 5 V echo signal directly to ESP32 GPIO. |
| 5V / V5 rail | HC-SR04 VCC | Use the correct regulated supply and common ground. |
| GND | Common ground as required | ESP32, sensor, and L298N logic ground must share a reference. |
| ENA / ENB | L298N enable terminals | If jumper caps are fitted, PWM speed control may be bypassed; verify actual build. |
| OUT1–OUT2 | One DC motor | Check motor pairing against the module labels. |
| OUT3–OUT4 | Other DC motor | Check motor pairing against the module labels. |

# Power Arrangement (verify the actual build)

The described setup uses a power bank for the ESP32 boards.

The described setup uses a two-cell series Li-ion battery pack for motor power through the motor driver and switch.

Two 3.7 V nominal cells in series provide about 7.4 V nominal and about 8.4 V when fully charged.

Connect common grounds where required by the control circuit. Do not connect separate positive supply rails together unless the design explicitly requires it.

Confirm the L298N board's logic-supply jumper/regulator arrangement from its exact module documentation.

# ECHO Divider Caution

The supplied notes mention three 1 kΩ resistors for the ECHO divider. Do not assume any resistor arrangement is safe without checking the actual topology and measuring the GPIO-side voltage. A simple two-resistor divider from 5 V to approximately 3.3 V commonly uses a suitable resistor ratio; three equal 1 kΩ resistors may produce a different ratio depending on how they are connected. Draw and verify the actual divider before applying power.

# Pre-Power Checklist

Battery polarity checked with a meter.

No exposed wire strands touching adjacent pins.

ECHO level shifting verified.

Motor driver and ESP32 grounds connected as designed.

Wheels lifted for initial tests.

Physical power switch accessible.