# Setup and Run Guide

File: docs/setup.md

# 1. Before You Begin

Robot chassis with two DC motors and wheels.

ESP32 development board and ESP32-CAM with a compatible USB/programmer board.

L298N driver, HC-SR04 sensor, appropriate jumper wires, and the documented power sources.

Windows laptop with Arduino IDE, Python, and a working Wi-Fi network/hotspot.

The final firmware files and laptop_follow.py from the repository.

# 2. Hardware Assembly

Mount the motors, wheels, caster, controller boards, motor driver, and ultrasonic sensor securely.

Wire the motor outputs to the L298N and the L298N logic inputs to the ESP32 pins listed in hardware/pin_table.md.

Connect the HC-SR04 trigger and echo lines as shown in the pin table. The ECHO line must be level-shifted/divided before it reaches an ESP32 GPIO.

Use the motor battery only for the motor-driver motor supply, and use the specified stable 5 V supply for the boards if that is how the final build is designed.

Join required grounds. Check polarity and continuity before switching on.

Keep wheels lifted for first powered tests.

# 3. Arduino IDE Setup

Install Arduino IDE and the Espressif ESP32 board package using the Boards Manager.

For an AI Thinker-style ESP32-CAM, select the matching board profile only if it matches your exact module. The common profile is AI Thinker ESP32-CAM.

For the separate car controller, select ESP32 Dev Module if it matches your actual development board.

Select the correct COM port. If upload fails, check the USB cable, driver, board profile, boot mode, and upload speed.

Open the camera sketch, replace Wi-Fi placeholders locally, compile, and upload.

Open the car-controller sketch, replace Wi-Fi placeholders locally, compile, and upload.

Open Serial Monitor at the baud rate configured in each sketch and record the IP address printed by each device.

Board options and partition names vary by ESP32 core version. Use the options available in your installed Arduino IDE and confirm them against the board and sketch; do not blindly apply a setting that does not appear.

# 4. Wi-Fi Configuration

Set the same suitable Wi-Fi network credentials in both sketches if the architecture requires both devices to join the same network.

Use a 2.4 GHz network/hotspot where required by the ESP32 board.

Do not commit real Wi-Fi credentials to a public repository. Use placeholders in the public version.

Record the camera stream address and robot command endpoint after the boards connect.

If an address changes after reconnecting, update the laptop script configuration.

# 5. Python Environment

Install a supported Python 3 version. Prefer a stable release supported by the required packages; verify package compatibility before choosing a very new version.

Open a terminal in the folder containing laptop_follow.py.

Create and activate a virtual environment if possible.

Run: python -m pip install -r requirements.txt

If no requirements file exists yet, create one listing only packages imported by the script, such as ultralytics, opencv-python, and requests, after verifying the imports.

# 6. Configure the Laptop Script

Set CAM_URL to the actual camera stream URL. The supplied notes describe a stream endpoint like http://<camera-IP>:81/stream; verify the actual firmware's endpoint.

Set ROBOT to the actual controller IP or base URL expected by the script.

Confirm the command endpoint and parameter names match the ESP32 firmware.

Check that the selected model is YOLOv8n and that the script uses the person class, if that is what the actual code does.

# 7. Safe First Run

With motors disabled or wheels lifted, verify that the camera stream opens in a browser or OpenCV window.

Test the controller's command endpoint with the robot powered safely and wheels lifted. Only use the exact endpoint implemented by your firmware.

Test stop first, then individual directions for short periods.

Check the ultrasonic sensor independently and confirm the displayed or logged distance is plausible.

Run the laptop script and observe detections and commands before placing the robot on the floor.

Only perform a short floor test in a clear area with a person ready to disconnect motor power.

# 8. Shutdown

Send the stop command if available.

Switch off motor power before moving wires or adjusting the driver.

Disconnect power before changing circuit connections.

Record errors and changes so the setup can be repeated.

# 9. Version Record

| Item | Version used | How to verify |
| --- | --- | --- |
| Arduino IDE | Fill in actual installed version | Help → About |
| ESP32 board package | Fill in actual installed version | Boards Manager |
| Python | Fill in actual installed version | python --version |
| OpenCV | Fill in actual installed version | python -c "import cv2; print(cv2.__version__)" |
| Ultralytics | Fill in actual installed version | python -c "import ultralytics; print(ultralytics.__version__)" |